Design and Implementation of Security System Based on Microcontroller ATMega8535

Adris#1, Eko Patra T.W.#2, Arwin D.W. Sumari#*3

# Department of Electronics , Indonesian Air Force Academy Jl. Laksda Adisutjpto, Yogyakarta, 55002 – INDONESIA

1adris_effendy@yahoo.com 2den_patra@yahoo.co.id

*School of Electrical Engineering and Informatics, Bandung Institute of Technology Labtek VII Jl. Ganesha No.10, Bandung 40132 – INDONESIA

3 daemon00idaf@yahoo.com

Abstract - Security is very important matter for personnels as well as valuable material. For this reasons a security system is extreemly needed. In this paper, we propose design and implementation a security system based on microcontroller ATMega8535. For doubling protection the system uses object detection sensor and security password.  In the operation mode there will be 2 steps that will be carried out. At the first step, the sensor detects the present of an object then activated the system while waiting the security password to be entered. At the second step, system detects the correctness of security password, then if it is correct, the ATMega8535 microcontroller will drive signal to activated the gate system. This security system is implemented by using C programming language. The private identity and the security password is costumizable and can be designed for our own needs.

Keywords : ATMega8535 Microcontroller, Object Sensor, Security System , Security Password.

- INTRODUCTION

Security system is an important matter for very important installation not only for military but also for civilians. In this paper we propose design and implement ATMega8535 microcontroller based security system supported with double protection, namely object detection sensor and security password.

The paper is started with Section I which cover the background of the paper and continued with Section II which explain some foundation theories briefly. Section III describes design and implementation of ATMega8535 microcontroller based security system. Section IV covers some concluding remarks of the paper that has been presented.

- FOUNDATION THEORY

- Object detection sensor[2]

Object detection sensor uses optocoupler which is activated by the light intensity. Optocoupler consists of 2 parts namely transmitter and receiver. The transmitter uses infrared LED since it is better than the ordinary one. The sensor is activated when the transmitted light between transmitter and receiver is blocked by an object.

- Microcontroller[1]

ATMega8535 is 8-bit microcontroller with 8 Kbytes Flash ISP. This microcontroller needs only 1 clock cycle for executing an instruction. This is aimed to optimize the thread-of between power consumption and speed. The main components of ATMega8535 microcontroller are 3 timers/counters, 32-register CPU, 512 bytes of SRAM and USART port for serial communication.

- DESIGN AND IMPEMETATION OF SECURITY SYSTEM

- Design and Implementation of Hardware

Figure 1.  Block Diagram of ATMega8535 microcontroller based Security System

General system block diagram is depicted in figure 1. 4x4 keypad is used for input device. The probability of the password is  P = 2n, where n is total character of keypad. In this design, n=5 is choose for alphabet and number, yields 25 or 32 password combination.

The output is displayed on LCD. The L293 is choose as motor driver. Step down trafo as power supply will produces 7.5 volt on secondary output and become 5 volt after the LM7805 voltage regulator.

- Design and Implementation of Software

This system uses C language for software programming and CodeVisionAVR as compiler. For microcontroller programming, the source code is wrote in C language and compiled in CodeVisionAVR. For downloading to microcontroller ATMega8535, the STK200+/300 Kanda System or compatible is used.

The LCD is connected to port A by using Code VisionAVR, defined as follow.

// Alphanumeric LCD Module functions

#asm

.equ __lcd_port=0x12 ;PORTA

#endasm

#include <lcd.h>

Keypad is connected to port B.

#define KEYPAD_TELPAD // using a telephone style keypad

#define KEYPAD_USEPORTC // wish to connect keypad to PortC

#define KEYPAD_USEDISPLAY // wish to use and LCD for display

#include "keypad.h"

Flowchart for system operating is explained in figure 2.

Figure 2.  Flowchart of system operation

- CONCLUDING REMARK

The following are concluding remark obtained from system design, implementation and system validation.

- This system uses double security system to assure security level.

- Control technology by using computer programming and microcontroller yield a simple system operation and costumizable system.

REFERENCES

[1] Wardhana, Lingga (2006), Belajar Sendiri Mikrokontroller AVR seri ATMega8535 Simulasi Hardware dan Aplikasi, Andi Offset, Yogyakarta.

[2] http://www.forum.lareosing.org/archieve/ind-ex.php/f-5.html, sensor optocoupler, Downloaded on July 24th 2008 , at 19.30.

[3] http://www.iwan@elektro.ft.undip.ac.id, Tu-torial Mikrokontroller AVR Part1, Downloaded on July 26th 2008, at 20.00.

[4] http://www.datasheet.com.html, Datasheet ATMega8535, Downloaded on August 4th 2008 at 20.30.

[5] http://instruct1.cit.cornell.edu/courses/ee476/ keypadRW88/keypad.html, Keypad instruction, Downloaded on August 8th 2008 at 19.30.

[6] Bejo, agus (2008), C&AVR Rahasia Kemudahan Bahasa C dalam Mikrokontroller ATMega8535, Graha Ilmu, Yogyakarta.