Limited Speech Recognition for Controlling Movement of Mobile Robot Implemented on ATmega162 Microcontroller

Thiang, Dhanny Wijaya

Electrical Engineering Department, Petra Christian University

121-131 Siwalankerto Surabaya, Indonesia

Abstract-This paper describes about speech recognition implemented on ATmega162 microcontroller. The word (voice command) in a speech signal is applied for controlling movement of a mobile robot. The mobile robot will move according to the voice command in the speech signal. There are five commands, which are Indonesian language, used to control the movement of mobile robot. They are “maju”, “mundur”, “kiri”, “kanan”, and “stop” used to command the mobile robot to move forward, move backward, turn left, turn right and stop respectively.

The methods used in this research are Linear Predictive Coding (LPC) and Hidden Markov Model (HMM). LPC is used to extract word data from a speech signal. HMM is used to recognize the word pattern data, which are extracted from a speech signal. Sampling rate of the speech signal is 8 kHz and the speech signal is sampled for 0.5 seconds.

Experiments were done in several variations of observation symbol number and number of sample. The experimental results show that the highest recognition rate that can be achieved by the system is 87%. The mobile robot can move in accordance with the voice command that is given to the robot.

- Introduction

Automatic speech recognition by machine has been a goal of research for more than four decades. However, in spite of the glamour of designing an intelligent machine that can recognize the spoken word and comprehend its meaning, and in spite of enormous research efforts spent in trying to create such a machine, it is far from achieving the desired goal of a machine that can understand spoken discourse on any subject by all speakers in all environments [1].

The speech recognition system has also been implemented on some particular devices. Some of them are personal computer (PC), digital signal processor, and another kind of single chip integrated circuit. A framework to address the quantization issues which arise in fixed-point isolated word recognition was introduced in [2]. The system was developed using C++ language which is implemented on a PC. Reference [3] describes a speech recognition system using SPHINX-II, an off-the-shelf speech recognition package [4]. In reference [5] and [6], speech recognition system has been tried to be implemented on a FPGA and an ASIC.

In the previous work [7], the speech recognition system was implemented on MCS51 microcontroller. There are two approaches implemented to recognize the word in a speech signal. The first approach is Linear Predictive Coding (LPC) combined with Euclidean Squared Distance. The second approach is Hidden Markov Model (HMM)-simple segmentation and centroid value. The highest recognition rate that can be achieved is about 78.57%.

This paper describes further work, which introduces the speech recognition which was implemented on another microcontroller. The microcontroller where the speech recognition was implemented on is ATmega162. This microcontroller is faster than MCS51 microcontroller and also popular in Indonesia.

The method implemented in this speech recognition system is LPC and HMM. LPC is used as extraction method and HMM is used as the recognition method. There are five words used as the command for controlling movement of mobile robot. The number of words used as command is limited because the microcontroller has limit number of memory. The five words used as the commands are “Maju”, “Mundur”, “Kiri”, “Kanan”, and “Stop” in which used to command the mobile robot move forward, backward, turn left, turn right, and stop respectively. All those words are Indonesian language.

- Linear Predictive Coding

The basic idea behind the LPC model is that a given speech sample at time n, s(n), can be approximated as a linear combination of the past p speech samples, such that

s(n)≈a1s(n−1)+a2s(n−2)+⋅+aps(n−p)	(1)

where the coefficients a1,aa,...,ap are LPC coefficients and  p is LPC order.

The problem of the LPC analysis is to determine the set of LPC coefficients directly from the speech signal. Since the spectral characteristics of speech vary over time, we have to estimate the LPC coefficients from a short segment of the speech signal occurring around time n. This problem can be solved by using LPC processor.

The basic steps of LPC processor include the following [1][8]:

- Preemphasis: The digitized speech signal, s(n), is put through a low order digital system, to spectrally flatten the signal and to make it less susceptible to finite precision effects later in the signal processing. The output of the preemphasizer network, , is related to the input to the network, s(n), by difference equation:

s(n)=s(n)−as(n−1)	(2)

The most common value for ais around 0.95.

- Frame Blocking: The output of preemphasis step, s(n), is blocked into frames of N samples, with adjacent frames being separated by M samples. If xl(n) is the lthframe of speech, and there are L frames within entire speech signal, then

xl(n)=s(Ml+n)	(3)

where n = 0,1,…,N – 1  and  l = 0,1,…,L – 1

- Windowing: After frame blocking, the next step is to window each individual frame so as to minimize the signal discontinuities at the beginning and end of each frame. If we define the window as w(n), 0 ≤ n ≤ N – 1, then the result of windowing is the signal:

xl(n)=xl(n)w(n)		(4)

where 0 ≤ n ≤ N – 1

Typical window is the Hamming window, which has the form

w(n)=0.54−0.46cos2πnN−1

0 ≤ n ≤ N – 1 	(5)

- Autocorrelation Analysis: The next step is to auto correlate each frame of windowed signal in order to give

rl(m)=n=0N−1−mxl(n)xl(n+m)

m = 0,1,…,p 	(6)

where the highest autocorrelation value, p, is the order of the LPC analysis.

- LPC Analysis: The next processing step is the LPC analysis, which converts each frame of p + 1 autocorrelations into LPC parameter set by using Durbin’s method. This can formally be given as the following algorithm:

E(0)=r(0)	(7)

ki=r(i)−j=1i−1αji−1r(|i−j|)Ei−1	1 ≤ i ≤ p	(8)

αi(i)=ki	(9)

αj(i)=αj(i−1)−kiαi−j(i−1)	1 ≤ j ≤ i-1	(10)

E(i)=(1−ki2)Ei−1	(11)

By solving the equation 7 to 11 recursively for i = 1,2,…,p, the LPC coefficient, am, is given as

am=αm(p)	(12)

- LPC Parameter Conversion to Cepstral Coefficients: LPC cepstral coefficients, is a very important LPC parameter set, which can be derived directly from the LPC coefficient set. The recursion used is

cm=am+k=1m−1km⋅ck⋅am−k	1 ≤ m ≤ p	(13)

cm=k=m−pm−1km⋅ck⋅am−k	m > p	(14)

- Speech Recognition Using Hidden Markov Model

A. Design of HMM Structure

Type of HMM that was implemented in this speech recognition system is a left-right model which is also known as Bakis model. This model has the property that, as time increases, the state index increases or stays in the same state. But, in this speech recognition system, the HMM model is designed in such that the state index always increases as the time increases and never stays in the same state. And also, the state index always increases by one to the next state index as the time increases. Figure 1 shows the structure of the HMM that is used in the designed speech recognition system.

Fig 1. N States Left-Right HMM Model

By designing the HMM structure like this, the state transition probability distribution is fixed as:

aij=1for j−i=1	(15)

The initial state probability distribution is fixed because the initial state is always state 1. The state never starts with other states. So, the initial state probability distribution of this designed HMM structure is:

πi=1for i=1	(16)

The state of HMM is associated with index of LPC cepstral coefficient of the speech signal. Thus, the number of state is equal to number of dimension of LPC cepstral coefficients. The observation symbol of HMM is associated with magnitude of LPC cepstral coefficient of the speech signal.

B. Training system Using HMM

Block diagram of HMM training system is shown at figure 2. The main task of the HMM training system is to create HMM λ model of each word sample. Parameters of HMM λ model consist of number of state (N), number of observation symbol (M), state transition probability distribution (A), observation symbol probability distribution (B), and initial state probability distribution (π). By using HMM structure shown in the figure 1, all the parameters are known except the observation symbol probability distributions (B) which vary depending on the training data set. Therefore, in this particular HMM training system, only observation symbol probability distributions in the state are calculated. The others parameters are pre-defined.

Firstly, the speech signal is processed by using LPC processor and result the LPC cepstral coefficients. Vector quantization is applied to the each cepstral coefficients and result the observation symbol of each state. The sequences of observation symbol of state 1, state 2, up to state N represent the observation sequence of the word.

Fig 2. Block Diagram of HMM Training System

If number of samples per word that are trained in the HMM speech recognition system is R, and each sample is converted to the observation sequence by the feature extraction and vector quantization process, the observation symbol probability distribution (B) in a state can be calculated by using following equation:

bjk=njvkR	(17)

where bjk is probability of observation symbol vk in the state j, njvk is number of observation symbol vk in the state j, and R  is number of sample.

C. Recognizer System Using HMM

Fig 3. Block Diagram of HMM Recognizer System

Block diagram of HMM recognizer system is shown at figure 3. Same as in the HMM training system, firstly, the speech signal of an unknown word is converted to the observation sequence of that word. This is done by performing the LPC processor and vector quantization process. Then, the observation sequence probability of unknown word for a given model λ (PO|λ) is calculated for each HMM word model. The probability of observation sequence calculation is done by using the forward procedure method. By applying the parameter A and π of the designed HMM (15) and (16) to the forward procedure equations, the probability of the observation sequence, O = (o1 o2 … oN), for the given model can be calculated by using the following equation:

PO|λ=b1o1⋅b2o2⋅b3o3 ⋯ bNoN	(18)

where  bNoN is probability of N-th observation symbol ( oN)in state N.

Equation 18 is used to calculate the observation sequence probability of unknown speech signal and the unknown speech signal will be recognized as HMM word model which has the maximum probability of observation sequence (PO|λ).

- Experimental result

Experiments were done in several variations of number of observation symbol. The number of observation symbol used in this experiment is 10 observation symbols and 20 observation symbols. The experiments were also done in various numbers of samples as the data training set. The experiments were done using 5 samples, 10 samples, 15 samples, and 20 samples.

Table I shows experimental result of the system using 10 observation symbols and the system is trained using 5 samples, 10 samples, 15 samples and 20 samples. Table II shows experimental result of the system using 20 observation symbols and the system is also trained using 5 samples, 10 samples, 15 samples and 20 samples.

TABLE I

Summary of Experimental Result of Speech Recognition System Using 10 Observation Symbols

| Word | Recognition Rate (%) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 5 samples | 10 samples | 15 samples | 20 samples |
| Maju | 75 | 75 | 55 | 75 |
| Mundur | 95 | 90 | 65 | 55 |
| Kanan | 75 | 40 | 45 | 45 |
| Kiri | 95 | 100 | 95 | 85 |
| Stop | 80 | 75 | 75 | 60 |
| Average Rec. Rate | 84 | 76 | 67 | 64 |

TABLE II

summary of Experimental Result of Speech Recognition System Using 20 Observation Symbols

| Word | Recognition Rate (%) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 5 samples | 10 samples | 15 samples | 20 samples |
| Maju | 85 | 60 | 70 | 50 |
| Mundur | 100 | 100 | 65 | 75 |
| Kanan | 75 | 40 | 25 | 50 |
| Kiri | 100 | 65 | 75 | 80 |
| Stop | 75 | 85 | 70 | 70 |
| Average Rec. Rate | 87 | 70 | 61 | 65 |

From table I and table II, we can see that the highest recognition rate that can be achieved by the system which is trained using 5 samples. And overall, the highest recognition rate that can be achieved by the system is 87%. This recognition rate is achieved by the system using 20 observation symbols and 5 samples for training.

Comparing with the previous work, the highest recognition rate that can be achieved is 78.57% [7]. By changing the recognition method from Euclidean Squared Distance to HMM, the system has improvement in the recognition rate.

From table I and table II, we can also see that increasing the number of sample does not always improve the recognition rate. And also, increasing the number of observation symbol does not always improve the recognition rate.

From the experiment, we also got that average time recognition an unknown word using LPC-HMM is about 15 to 17 seconds. This result is faster than the result of previous work [7], but, it still too slow.

- Conclusion

From the experiment result, it can be concluded that the speech recognition has successfully been implemented on ATmega162 microcontroller. The highest recognition rate that can be achieved is 87%. This result is achieved by the system using 20 observation symbols and 5 samples per word for data training.

Comparing with the result of previous work, this speech recognition system gives a better recognition rate. Thus, it can be concluded that HMM method is better than Euclidean Squared Distance.

For further work in order to get better time response, the system must be implemented on a faster microcontroller or digital signal processor.

References

- Lawrence Rabiner, and Biing Hwang Juang, Fundamentals of Speech Recognition. Prentice Hall, New Jersey, 1993

- Y.M. Lam, M.W. Mak, and P.H.W. Leong , “Fixed point implementations of Speech Recognition Systems”. Proceedings of the International Signal Processing Conference. Dallas. 2003

- Soshi Iba, Christiaan J. J. Paredis, and Pradeep K. Khosla. “Interactive Multimodal Robot Programming”. The International Journal of Robotics Research (24), pp 83 – 104, 2005

- Huang, X. et al. “The SPHINX-II speech recognition system: an overview”. Computer Speech and Language 7(2): pp 137–148, 1993.

- Treeumnuk, Dusadee. (2001). Implementation of Speech Recognition on FPGA. (Masters research study, Asian Institute of Technology, 2001). Bangkok: Asian Institute of Technology.

- Sriharuksa, Janwit. (2002). An ASIC Design of Real Time Speech Recognition.  (Masters research study, Asian Institute of Technology, 2002). Bangkok: Asian Institute of Technology.

- Thiang, “Implementation of Speech Recognition on MCS51 Microcontroller for Controlling Wheelchair”, Proceedings of International Conference on Intelligent and Advanced System. Kuala Lumpur, Malaysia, 2007.

- Ethnicity Group. “Cepstrum Method”. 1998 http://www.owlnet.rice.edu/~elec532/PROJECTS98/speech/cepstrum/cepstrum.html