Simulate an All-Digital Frequency-Comparator Circuit Using PSpice/PLogic

This article illustrates how a hierarchical all-digital design with two

implementation views, can be defined in Schematics, and subsequently

simulated in PSpice/PLogic.  This discussion is relevant to version 5.2 and

later of the Design Center.  The example circuit is a basic frequency-compar-

ator (see Figure 1).  All parts used in the schematic are provided in the

standard Symbol and Model Libraries.  One implementation is chosen for PSpice/

PLogic simulation to demonstrate the circuit's behavior.

The frequency-comparator circuit accepts two reference frequency inputs, and

a test frequency input which is compared to the references. After initial-

ization and start-up, the circuit produces fast, slow, OK, and error indica-

tions. Operation is continuous as long as both of the reference signals are

applied.

Initialization is accomplished by applying a low pulse to the INIT input,

having a minimum width of 40 nsec.  At least 40 nsec after the negative-going

edge of the INIT input, circuit operation commences upon applying a negative-

going edge to the RUN input.

Outputs of the circuit-SLOW, FAST, OK, and ERROR-are pulses indicating the

result of comparing the test frequency signal, FTEST, to the low and high

frequency reference signals, REFL and REFH, respectively.   The ERROR pulse

is generated if more than 7 complete periods of the REFL signal are observed

with no activity on the FTEST input during that time.

Implementation

The frequency-comparator circuit is designed in Schematics using hierarchical

blocks (logic shown on these facing pages) for the initializer (INIT block),

cycle-detectors (PICD blocks), state-decoder (SDL block), and error-detector

(EDL block).  The design has two alternative implementations:  a gate-level

implementation using off-the-shelf 74xx parts (see Figure 5), and a function-

ally equivalent implementation using a mixture of 74xx parts and a commonly

available Programmable Array Logic (PAL) device, PAL20RP4B (see Figure 6).

Both implementations use the digital stimulus include file "frqchk.stm",

providing definitions for the INIT, RUN, MODE, REFH, REFL, FTEST and SYSCLK

input signals (see Figure 9 on page 5).

The design alternatives are implemented as two views of the SDL block, with

the DEFAULT view being the gate-level implementation, and the PAL-IMPL view

being the PAL implementation.  For the PAL-IMPL view, the data required to

program the PAL20RP4B device is supplied in a JEDEC file, "frqchk.jed,"

generated using OrCAD/PLD (see Figure 8 on page 5).

Operation

The three frequency inputs-REFL, REFH, FTEST-each drive a separate instance

of a cycle-detector circuit (PICD blocks).  Each cycle-detector is made up

of two pairs of D-type flip-flops and a few basic gates.

After having been "reset" and "enabled," the cycle-detectors output a HI

level as soon as two similar edges (e.g., falling) have been applied.  This

indicates that one complete period of the input signal has been observed.

The circuit implements a simple finite-state machine (see Figure 7) that

recognizes the order in which the individual frequency inputs make complete

cycles.  For example, suppose that the REFH signal period is observed first

(generating N1), followed by the REFL signal period (generating N2), then

the FTEST period (generating N0). This indicates that the FTEST frequency is

too low and that the SLOW output should be pulsed.   But if the FTEST period

is observed before the REFL cycle, an OK pulse is produced.   The state

machine "current state" simply represents the order of activity that has been

observed since the last "initialization" or "reset," which occurs every time

any kind of output pulse is generated.

The cycle-detectors monitor the input activity and produce the "next state"

value (N3, N2, N1, N0), which is fed to the state-decoder (SDL block).  At a

rate determined by the system clock, SYSCLK, this "next state" becomes the

"current state"; the 74154 4/16 decoders in the gate-level view of the state-

decoder, continually provide unary logic indications of the next/current

"transitions" (since "next state" values are not synchronized to SYSCLK).

The random combinational logic in this same view recognizes the specific

transitions that comprise the conditions of interest, i.e., FAST, SLOW, and

OK, as per the state-transition diagram.  (In the PAL view, the PAL20RP4B

device replaces all of the decoding logic as well as the 4-bit register

representing the "current state" value.   The alternative implementations are

functionally identical.)  Note that the output indicators are not "static"

state assignments; they are derived from selected state transitions.  Thus,

S14 -> S15 recognizes a SLOW condition, while S10 -> S15 signifies an OK

condition.

The error-detector logic (EDL block) waits for the TIMEOUT signal output by

the timeout-generator.  The timeout-generator is simply a counter whose Q3

output indicates that the 8th rising edge of the low frequency reference,

REFL, has occurred.  If none of the normal output indicators (SLOW, FAST, or

OK) have occurred before TIMEOUT, the ERROR output is asserted.  The error

detector also asserts its DONE output whenever any of FAST, SLOW, OK, or

ERROR have occurred.

The initialization/reset logic (INIT block) performs two functions.  One

distributes the effects of the INIT and RUN inputs, as defined in the

stimulus include file, "frqchk.stm" (see Figure 9).  The other uses the DONE

signal from the error-detector to generate a RESET pulse; this has the same

effect as the external RUN pulse-to restore the state machine to its starting

state (0) as well as reset the cycle-detectors, timeout generator, and flip-

flops in the error-detector.  Normal operation then resumes.

PSpice/PLogic Simulation-PAL View

To simulate the PAL implementation of the frequency-comparator circuit, these

steps must be followed:

(1)	The transient analysis must be enabled under the Analysis/Setup

command.  The transient analysis is defined with:  Print Step = 1us,

Final Time = 1ms.

(2)	All flip-flops must be initialized in the 0 state (rather than the

default X state).  This is set with the DIGINITSTATE option under

the Analysis/Setup/Options command.  This allows the simulator to

properly initialize the circuit by forcing the reset logic to a

deterministic state (non X; the hardware implementation would

eventually sync itself to the input stimuli and operate correctly).

(3)	In the top-level schematic, the SDL block is the only block with more

than one view.  Without further setup, Schematics will generate the

PSpice/PLogic netlist using the DEFAULT gate-level view for SDL.

To use the PAL view instead, the PAL-IMPL view name must be specified

in the View field of the Translators dialog (under the

Configure/Translators command).  After running 	the simulation by

selecting Analysis/Simulate, the state-machine operation is viewed

in Probe by placing markers on the appropriate wires and buses

(using the Markers/Mark Voltage/Level command), or by typing the sig-

nal names in the Probe dialog under the Trace/Add command as follows:

SYSCLK, REFH, REFL, FTEST

FAST, SLOW, OK, ERROR

{N3, N2, N1, N0};NEXT

{C3, C2, C1, C0};CURRENT

Figure 10 demonstrates correct response of the circuit to the digital

stimulus at FTEST. If you are currently running version 5.3 of the

Design Center-System 3 (under Microsoft Windows or Sun OpenWindows),

and would like a copy of the schematic files for the frequency-

comparator circuit, please contact MicroSim Technical Support.